### Validation and Rigorous Evidence: AttentionSE Autoencoder vs. Linear ROM

This document provides a technical summary and analysis of the three key pieces of evidence supporting the use of the AttentionSE (Squeeze-and-Excitation) Autoencoder for fluid dynamics velocity reconstruction. We evaluate its performance against the industry-standard linear baseline, Proper Orthogonal Decomposition (POD), also known as Principal Component Analysis (PCA).

#### 1. Compression Efficiency and the “Elbow” Point (Ablation Study)

Artifact: Documentation/ablation_study.png

In the literature of Reduced Order Modeling (ROM), a critical question is how many degrees of freedom (latent dimensions) are required to capture the essential dynamics of the flow. Traditional methods like POD/PCA provide a linear approximation, while non-linear autoencoders can represent the data on a curved manifold, theoretically requiring fewer dimensions for the same accuracy.

- The “Elbow” Discovery: Our ablation study sweeps through latent dimensions z∈[1,256]. We observe a clear “elbow” in the RMSE curve near z=40 to z=64. This indicates a point of diminishing returns where additional dimensions do not significantly improve reconstruction accuracy.

- Selection of z=47: Our chosen dimension of 47 (a compression ratio of approximately 8:1 from the original 375 dimensions) sits squarely on the conservative side of this elbow. By selecting a dimension near the elbow, we ensure that we are not over-compressing (which would lose physical features) nor under-compressing (which would defeat the purpose of the ROM).

- Justification: Studies like Murata et al. (2020) have shown that non-linear autoencoders can achieve the same reconstruction error as POD while using 30-50% fewer latent dimensions. Our results confirm this efficiency, as the AttentionSE model consistently outperforms PCA at the same dimension.

#### 2. Comparison to Linear ROM (PCA/POD)

Artifact: Documentation/ROM_Comparison_AE_vs_PCA.png

Reviewers in fluid mechanics (e.g., Journal of Fluid Mechanics) often question the necessity of a neural network if a linear POD can capture the flow. Our head-to-head comparison shows that the AttentionSE architecture provides a 7-15% reduction in RMSE compared to a POD model with the same number of components.

- The Non-Linear Advantage: While POD is optimal for linear reconstruction, fluid flows are inherently non-linear. The Squeeze-and-Excitation (SE) blocks in our model allow the network to perform “channel-wise feature recalibration,” effectively prioritizing which velocity components are most critical for the local flow structure (vortices, gradients).

- Robustness: The performance gap is most pronounced when reconstructing complex, turbulent regions where linear superpositions (POD modes) tend to “blur” features.

#### 3. Physical Consistency and Gradient Fidelity

Artifacts: Documentation/divergence_comparison.png, Documentation/vorticity_fidelity.png

RMSE alone is a poor metric for fluid dynamics because it does not account for the physical laws governing the flow. A “good” reconstruction must respect mass conservation and preserve the energy spectrum (gradients).

- Divergence-Free Condition (∇⋅u≈0): Incompressible flow must satisfy the continuity equation. Our analysis shows that the AttentionSE model produces lower divergence errors than the PCA baseline. This implies that the neural network has implicitly “learned” the physical constraint of incompressibility from the data, a phenomenon often noted in Physics-Informed Neural Networks (PINNs) literature (Raissi et al., 2019).

- Vorticity and Gradients (ω=∇×u): Vorticity represents the “spinning” or small-scale eddy structures in the flow. Linear POD often acts as a low-pass filter, smoothing out these high-frequency gradients. Our vorticity error distribution confirms that the Autoencoder preserves high-gradient regions significantly better than PCA, maintaining the “sharpness” of the flow field.

### Conclusion

The evidence suggests that the AttentionSE Autoencoder is not just a statistical improvement over PCA, but a physically more consistent ROM. By selecting a latent dimension of 47, we strike a balance between aggressive compression and conservative physical preservation, ensuring that the model is robust enough for downstream tasks (like flow prediction or super-resolution) while remaining more efficient than traditional linear methods.

### Key Citations

- Murata, T., Fukami, K., & Fukagata, K. (2020). Nonlinear mode decomposition with convolutional neural networks for fluid dynamics. Journal of Fluid Mechanics.

- Hu, J., Shen, L., & Sun, G. (2018). Squeeze-and-Excitation Networks. CVPR.

- Raissi, M., Perdikaris, P., & Karniadakis, G. E. (2019). Physics-informed neural networks. Journal of Computational Physics.

- Guastoni, L., et al. (2021). Convolutional-network models to predict wall-bounded turbulence. Physical Review Fluids.