### Methods: Data Generation and Pre-processing

#### Experimental Setup and Data Acquisition

Flow velocity data were generated at the Fluid-Structure Interaction (FSI) Lab at the University of Massachusetts Dartmouth. Quantitative flow measurements were acquired using a Time-Resolved Volumetric Particle Tracking Velocimetry (TR-PTV) system. The experimental setup utilized a LaVision configuration consisting of: * Illumination: A 300 × 100 mm² LED array (FLASHLIGHT 300). * Imaging: A four-camera Minishaker box equipped with 8, 12, and 16 mm focal length lenses. * Synchronization: Cameras and illumination were triggered simultaneously via a LaVision Programmable Timing Unit (PTU). * Data Acquisition: High-speed video was acquired using Sony IMX252LLR/LQR CMOS cameras (2048×1088 resolution) and recorded uncompressed via a Core DVR.

The raw image sequences were processed using the Shake The Box (STB) algorithm within the DaVis 10 software environment. This enabled time-resolved, three-dimensional, three-component (3D-3C) Lagrangian particle velocity measurements ⟨vx,vy,vz⟩ within the volume.

#### Data Preparation and Quality Assurance

Prior to model training, the raw Lagrangian data underwent a multi-stage pre-processing pipeline to ensure spatial consistency and numerical stability:

- Health Assessment: Each data file was audited to verify the presence of a complete temporal sequence (T={1,…,1200}) and consistent row counts.

- Statistical Analysis of Extremes: Global extrema for all velocity components (vx,vy,vz) were determined to establish bounds for normalization.

- Data Type Correction: To optimize computational efficiency, spatial/temporal coordinates were cast to 32-bit integers, and velocity components to 32-bit floats.

- Normalization: Velocity components were scaled to a [0,1] range using a linear transformation based on global extremes (approximately −1.98 to 2.64 m/s), ensuring stable convergence during neural network training.

#### Spatial Mapping and Cubic Reconstruction

To transition from discrete point measurements to a structured representation suitable for deep learning, the data were reorganized into local volumetric neighborhoods.

As illustrated in Figure 1 (experimental_box.png), a sampling volume was defined by trimming the edges of the total measurement domain. This trimming ensures that every selected centroid possesses a complete surrounding neighborhood.

For each valid centroid, a 5×5×5 coordinate neighborhood was established (Figure 2, mapping_visualization.png). This neighborhood consists of the target centroid and its 124 nearest neighbors across the grid. The 3D-3C velocity components for all 125 points were gathered and flattened into a 375-dimensional feature vector V∈R375 (125 points × 3 velocity components). These “velocity cubes” capture the local flow structure, providing the high-dimensional input required for manifold learning.

### Visualizations and Artifacts

The following high-resolution artifacts (1200 DPI) have been generated in the Documentation/ directory:

- Documentation/experimental_box.png: A 3D schematic of the TR-PTV measurement domain, identifying the “Valid Sampling Region” (interior volume) and centroids used for data extraction.

- Documentation/mapping_visualization.png: A 3D breakdown of the 5×5×5 sampling neighborhood, illustrating the mapping of 125 spatial points to the 375-dimensional feature vector V.

The generating script, generate_mapping_viz.py, is available in the project root and is configured for publication-quality output.